# 📘 Networking Fundamentals — Point-wise Notes

## 🔹 What is an IP Address?

- IP address = unique identifier for every device in a network

- Used to:

  - Identify a device

  - Track activity

  - Allow/block access

  - Monitor traffic

- Needed because multiple devices exist on same network (home, school, office)

## 🔹 Why do devices need unique addresses?

- To control access (example: block a kid from Instagram)

- To track which device accessed a website

- To audit payment activity

- To secure the network

## 🔹 What does an IP address look like?

- Standard format: x.x.x.x

- Example: 192.168.0.1

- Each block (octet) = 0 to 255

## 🔹 IPv4 Standard

- Most commonly used

- Format: A.B.C.D

- Example ranges:

  - 172.16.0.34

  - 10.1.2.4

## 🔹 Why 0 to 255?

- Each block = 8 bits = 1 byte

- IP address = 4 bytes = 32 bits

- 8 bits maximum number = 255
(because 11111111 in binary = 255)

## 🔹 How computer reads IP address?

Example:
192 = 11000000 in binary
Why?

- Binary values = powers of 2

- 128 + 64 = 192

## 🔹 Total IPs available in IPv4

- 8 bits each block

- 4 blocks

- Total = 2^32 addresses

# 🔹 Subnet (What & Why?)

### Problem

If one device in a network is hacked, others may be compromised because all share same network.

### Solution

Split network into smaller secure networks = subnetting

### Benefits

- Security

- Isolation

- Privacy

# 🔹 Types of Subnets

### Public Subnet

- Access to internet

### Private Subnet

- No direct internet access

# 🔹 Why CIDR?

CIDR tells:

- How many IPs a subnet contains

- Which range belongs to which subnet

# 🔹 CIDR Notation Format

Example:

172.16.3.0/24

# 🔹 CIDR Capacity Calculation Trick 🔥 (Very Important)

Formula:

Number ofIPs=2^(32 - mask)

Examples:

/24 → 32-24 = 8 → 2⁸ = 256 IPs
/27 → 32-27 = 5 → 2⁵ = 32 IPs
/29 → 32-29 = 3 → 2³ = 8 IPs

# 🔹 Common CIDR Ranges

| CIDR | Networks |
| --- | --- |
| /8 | Very large |
| /16 | Medium |
| /24 | Small |

# 🔹 CIDR Class concept

| Class | CIDR | Size |
| --- | --- | --- |
| Class A | /8 | Huge |
| Class B | /16 | Medium |
| Class C | /24 | Small |

# ⭐ How to choose subnet size?

- Need 256 IPs → use /24

- Need 32 IPs → use /27

- Need 2 IPs → use /31

Simplest rule:

Remaining bits → power of 2 = available IPs

# 💡 Why Subnetting matters in Cloud?

Used in:

- AWS VPC

- Azure VNets

- Kubernetes networks

- On-prem networks

# 🧠 Most Important Takeaways

- IP is a unique identifier

- IPv4 = 4 octets

- Each octet = 8 bits = 0–255

- Subnet = security isolation

- CIDR = how many IPs per subnet

- IPs = 2^(32-mask)

# 🧠 Networking Fundamentals – OSI Model (Simple Notes)

## ⭐ Before OSI Model – Two things happen first

### 1️⃣ DNS Resolution

- You typed www.google.com

- Browser/router checks DNS to find its IP address

- Checks:

  - Browser cache

  - Router cache

  - ISP DNS

- Only when name resolves → data transfer starts
Example: google.com → 8.8.8.8

If DNS fails → request won’t start.

### 2️⃣ TCP 3-Way Handshake

Why?
To confirm server is ready to accept request.

Process:

- SYN → client says “hi”

- SYN-ACK → server replies “hi”

- ACK → client says “ok”

Only after that the real data starts flowing.

# ⚡ OSI Model – 7 Layers (Top → Bottom)

We always remember:
A P S T N D P
(Application, Presentation, Session, Transport, Network, Data link, Physical)

## Layer 7 – Application

- Browser creates request

- Protocol chosen (HTTP/HTTPS)

- We add request headers, authentication etc.

## Layer 6 – Presentation

- Encryption happens (SSL/TLS)

- Data formatting

- HTTPS = encrypted HTTP

## Layer 5 – Session

- Creates session between browser ↔ server
Why?

- So you don’t log in every time
Example: Facebook keeps you logged in
Session lives in cookies

## Layer 4 – Transport

- Data segmentation (splitting)

- Protocol for sending data defined:

  - TCP (reliable)

  - UDP (faster, no guarantee)

Example:

- HTTP → TCP

- DNS → UDP

## Layer 3 – Network

- Router adds IP addresses

  - Source IP

  - Destination IP

- Packets are created

- Routing happens (find shortest path)

## Layer 2 – Data Link

- Data converted into frames

- MAC address is added
Switches understand frames

## Layer 1 – Physical

- Bits (electrical signals)

- Travels on cables / fiber optics

# 📌 How data travels

Laptop → Browser

Layer7 → Layer1

Internet via:

- Router

- Switch

- ISP

- Backbone network

- Google servers
(through multiple hops)

At Google side:

Layer1 → Layer7

reverse process

# 📦 TCP/IP Model (Modern Model)

OSI (7 layers) becomes TCP/IP (4 layers)

- Application = L7+L6+L5

- Transport = L4

- Internet = L3

- Network access = L2 + L1

# 🎯 For DevOps Engineers

- OSI knowledge = good to have

- Not required day-to-day

- Helps when debugging network issues

- Helps when troubleshooting:

  - DNS

  - HTTP

  - SSL

  - TCP connectivity

  - Load balancers

  - Firewalls

# 🎁 Super Short Summary

| Layer | Name | What happens |
| --- | --- | --- |
| 7 | Application | HTTP request |
| 6 | Presentation | Encryption |
| 5 | Session | Maintain login |
| 4 | Transport | TCP/UDP splitting |
| 3 | Network | IP routing |
| 2 | Data Link | MAC + frames |
| 1 | Physical | Electrical signals |